## Safe work method statement (SWMS): Tree felling

Note: Work must be performed in accordance with this SWMS. This SWMS must be kept accessible for each relevant worker and available for inspection until the high risk construction work related to this SWMS is completed. If the SWMS is revised, every version should be kept. If a notifiable incident occurs in relation to the high risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident.

| Company name: |  | Principal contractor (PC): |  |
| --- | --- | --- | --- |
| Work activity: | Tree felling | Date SWMS provided to PC: |  |
| Works manager: |  | Workplace location: |  |

| High risk construction work: |  |
| --- | --- |
| ☐ Risk of a person falling more than 2 metres | ☐ Work on a telecommunication tower |
| ☐ Demolition of load-bearing structure | ☐ Likely to involve disturbing asbestos |
| ☐ Work in or near a confined space | ☐ Temporary load-bearing support for structural alterations or repairs |
| ☐ Use of explosives | ☐ Work in or near a shaft or trench deeper than 1.5 m or a tunnel |
| ☐ Work on or near pressurised gas mains or piping | ☐ Work on or near chemical, fuel or refrigerant lines |
| ☐ Work on or near energised electrical installations or services | ☐ Work in an area that may have a contaminated or flammable atmosphere |
| ☐ Tilt-up or precast concrete elements | ☐ Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |
| ☐ Work in an area with movement of powered mobile plant | ☐ Work in areas with artificial extremes of temperature |
| ☐ Diving work | ☐ Work in or near water or other liquid that involves a risk of drowning DELETE THIS BOX ONCE READ: These ArbSafe Pack resources are templates only. They must be customised for your workplace ensuring that: - Relevant legal requirements have been met, - Workplace / task specific risks are identified and managed, and - Workers are consulted with during the customisation / review process. |

| Person responsible for ensuring compliance with SWMS: | Example: Supervisor / Crew Leader | What measures are in place to ensure compliance with SWMS? | Examples: JHA before starting work On site supervision Regular spot checks and site inspections |  |  |
| --- | --- | --- | --- | --- | --- |
| How will SWMS control measures be reviewed? | Example: Control measures will be reviewed by the Site Supervisor / Crew Leader (in consultation with workers): At the start of each shift during the JHA ¦ If there is a change in the scope of work or work conditions ¦ After any incident or near miss (as required) ¦ At the request of an HSR or worker ¦ If the current controls are found to be ineffective |  |  |  |  |
| Approver/s: |  | Signature: |  | Approval Date: |  |

| What are the tasks involved? | What are the hazards and risks? | What are the control measures? |
| --- | --- | --- |
| List the work tasks in a logical order. | Identify the hazards and risks that may cause harm to workers or the public. | Describe what will be done to control the risk. What will you do to make the activity as safe as possible? |
| Assess tree and plan felling | Unidentified risk impacting felling | - Pre-felling site inspection and tree assessment - Assess / check environmental conditions (wind direction / strength, slope) - Ensure the tree fall path is free of structures / other trees / powerlines (outside the SADs) - Check your path of retreat / escape route (45 degrees both direction behind the tree) - Determine if felling will require assistance of pull rope, winch, extra wedges - If the tree is not safe to fell call for an EWP or Climber to dismantle, or postpone until better conditions - Standard PPE + chainsaw chaps / pants + hearing protection + work gloves + hard hat |
| Establish exclusion zone | Unauthorised access, injury to bystanders | - 2 x tree height exclusion zone (EZ) around site - Use cones / signage / barriers to restrict access - Spotters to actively monitor site |
| Chainsaw operation | Kickback, lacerations | - Ensure saw bar is long enough for selected cut - Pre-use inspection (chain sharpness, brake, chain tension, guards) - Competent / trained operator - Use of correct techniques (inc. safe stance, two-handed use) - Never operate above shoulder height |
| Tree Felling | Uncontrolled tree fall, falling branches – struck by tree or branches | - Assess lean, wind direction, and defects (cracks, rot, dead limbs) - Plan direction of fall - Check your path of retreat / escape route (45 degrees both direction behind the tree) - Put all required tools in a safe and reachable spot - Ensure a clear area at the base of the tree (clear lower branches to provide access) - Use wedges or mechanical assistance (winch or rope) if required (attach before you start felling) - Ensure exclusion zone is clear before you start felling - Notch/back cut correctly - Mark out locations on the tree trunk prior to cutting (scarf location and back cut position) - Check intermittently whilst cutting the scarf that the scarf is positioned correctly - Ensure back cut is positioned above the bottom of the scarf and below the middle point of the scarf |
| Pulling tree with rope or winch | Uncontrolled tree fall, falling branches – struck by tree or branches | - Pre-use equipment inspection - Check rope is long enough to reach tree, and so crew can stand at a safe distance - Suitable escape routes identified for crew - Visual / verbal communications between ground crew and feller - Remove rope slack before felling. Maintain a steady amount of tension during pull - DO NOT rock the stem back and forwards as this can snap the hinge or cause the tree to sit back – always pull steadily and smoothly - If the tree doesn’t go, gently release the pressure and allow the feller to adjust the hinge before trying again |
|  | Snapped pull or winch rope | - Pre-use equipment inspection - Maintain a steady amount of tension during pull - DO NOT over tension the rope - Visual / verbal communications between ground crew and feller to adjust as needed |
|  | Entanglement in pull rope or rope burn | - DO NOT wrap pull rope around hands or body, hold only in the palms of your hands - Ensure end of the rope is not coiled around feet or laying where it can be a trip hazard |

| Sign on Sheet (review and acknowledgement) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | Signature | Date |  | Name | Signature | Date |

Additional Information